## Project Report: Milestone 2

Project Name: TraceFinder - Scanner Forensics

Objective: Extracting and analyzing hand-crafted digital fingerprints to identify source scanners.

### 1. Data Preparation & Visualization

Before training the models, we had to ensure the scanner's "noise" was visible and independent of the document being scanned.

- Noise Extraction: We used a Gaussian Blur filter to separate the document content from the sensor noise.

- Result: By subtracting the blur from the original image, we created "Noise Maps."

- Key Finding: Visual comparison showed that the noise pattern (digital dust) remains the same for a specific scanner, whether it is scanning a Wikipedia page or an Official document.

### 2. Feature Engineering (The "Feature Bank")

We converted images into a table of 13 specific mathematical values (features) that describe the scanner hardware:

- Texture (LBP): Captured the micro-patterns and "grain" of the scanner sensor.

- Frequency (FFT): Captured hidden periodic patterns, likely caused by the scanner's motor vibration.

- Noise Variance: Measured the "sharpness" and intensity of the digital noise.

### 3. Model Performance & Evaluation

We trained three different Machine Learning "brains" to see which one could identify the scanners best based on these 13 numbers.

The Confusion Matrix Analysis:

- Success: The models can perfectly identify unique hardware (like Scanner 8).

- Confusion: There is significant "overlap" between identical scanner models (e.g., two Canon 120s). The model sees the "family resemblance" but cannot always tell the "siblings" apart.

### 4. Identified Limitations

While Milestone 2 was successful, we identified three major limitations of using "hand-crafted" features:

- Human Bias: We chose LBP and FFT, but there are likely microscopic pixel relationships that we missed.

- Model Ceiling: The accuracy reached a plateau because the features aren't deep enough to separate identical scanner units.

- Content Sensitivity: Despite our best efforts, very dark text still slightly interferes with the hand-crafted noise scores.

### 5. Conclusion & Next Steps

Milestone 2 proves that scanner fingerprints exist and are measurable.